Fanvil i10S/i10SV/i10SD SIP Mini Intercom

| The i10S/i10SV/i10SD SIP mini intercom is designed for indoor scenes with IP54 waterproof and dustproof. Supports wall mounting installation. It combines security, audio/video intercom and broadcasting functionalities and offers the qualified communication solution for users. Highlights Easy installation and configuration. IP54 waterproof and dustproof. -20 °C to 50 °C wide range temperature adaptability. Built-in 1W speaker and Acoustic Echo Canceller (AEC). Built-in 1short circuit input/output interface. Functionality including security, intercom and broadcasting. Built-in HD camera and with backlight compensation function (for i10SV). Adaptive 10/100 Mbps network port, integrated PoE Compatible with major platforms: Asterisk, Broadsoft, 3CX, Metaswitch, Elastix, Avaya etc. | i10S i10SV i10SD Model; Screen; Button; Camera. i10S; 10/100Mbps; 1; ×. i10SV; 10/100Mbps; 1; √. i10SD; 10/100Mbps; 2; ×. |
| --- | --- |

Generic

- 2 SIP lines

- Speed dial

- Linkage with IP camera to realize video intercom

- Oral announcement of the IP address(in English)

- Switch signal input and output

- Support programmable DSS key ports

- PoE enabled

Device Functions

- Auto-answering

- Programmable DSS keys

- Through web page volume adjustment

- Network time synchronization

- Time plan (task management)

- Dynamic multicast function

- Incoming call multicast

- HTTP API

- Action URL / Active URI

Call Functions

- Call out / answer / reject

- Call waiting

- Intercom, Multicast

- Speed dial

- Anonymous call (Hide caller ID)

Audio

- Narrowband codec: G.711A/U, G.729A/B

- Wideband codec: G.722, Opus

- Full-duplex hands-free speakerphone with AEC

- DTMF: In-band, Out-of-Band (RFC2833/ SIP INFO)

Video (Only i10SV)

- Sensor: 1/5” color CMOS

- Pixels: 2 Mega

- Video coding: H.264

- Resolution: 720P, 1080P

- Minimum illumination: 40%

- Viewing angle:

- Diagonal: 73°

- Horizontal: 63.3°

- Vertical: 49.6°

Network

- Physical: 10/100 Mbps Ethernet

- IP mode:IPv4/IPv6/IPv4&IPv6

- IP configuration: Static IP / DHCP / PPPoE

- VPN: L2TP / OpenVPN

Protocols

- SIP v1 (RFC2543), v2 (RFC3261) over UDP/TCP/TLS

- RTP/RTCP/SRTP

- STUN

- DHCP

- IPv6

- PPPoE

- L2TP

- OpenVPN

- SNTP

- FTP/TFTP

- HTTP/HTTPS

- TR-069

Deployment & Management

- Auto-provisioning via FTP/TFTP/HTTP/HTTPS/DHCP OPT66/SIP PNP/TR-069

- Web management portal

- Web-based packet dump

- Configuration export / import

- Firmware upgrade

- Syslog

Physical Specifications

- DSS Keys, follow model

- i10S/i10SV: 1 DSS key (speed dial key)

- i10SD: 2 DSS keys

- RJ45 port x1: connect the network

- WAN port: connect the network, support PoE

- Short circuit input and output port x1 (built-in relay)

- Relay: MAX DC30V / 1A, AC125V / 0.5A

- Port: terminal socket

- DC port: DC power input

- DC power input: DC12V / 1A

- Port: terminal socket

- PoE level: Class 3

- Protection level: IP54

- Housing material: ABS

- Installation: Wall-mounted

- Working temperature: -20℃ ~ 50℃

- Storage temperature: - 30℃ ~ 60℃

- Working humidity: 10~95%

- Device dimensions: 88x88x32 mm

- Gift box dimensions: 95x108x40 mm

- Outer CTN dimensions: 241x225x215 mm (20 PCS)

The above specifications may be updated in the future without prior notice. All hardware/software/physical features should be based on the final shipped products.

For more information, please visit: www.fanvil.com

Compatibility List:

www.fanvil.com

Add: 10/F Block A, Dualshine Global Science Innovation Center, Honglang North 2nd Road, Bao'an District, Shenzhen, China

Tel: +86-755-2640-2199   Fax: +86-755-2640-2618   Email: sales@fanvil.com